HARDWARE FOCUS – PROJECT NO. 7

Automated Verification Project

– Group 4 –

Nguyen Minh Duc – Tran Minh Tuan

# Introduction

The project requires two RISC-V RV32I processor implementations: a single-cycle processor and a 5-stage pipelined processor referenced from lab 6 and 7. The proposed project builds an automated verification environment using SystemVerilog to validate both implementations at the instruction, memory, control-flow, and debug-output levels.

## 1.1. Project Overview

- The single-cycle model is a Harvard-architecture RV32I core with separate instruction memory and data memory, designed for simplicity and one-instruction-per-cycle execution.

The pipelined model is a 5-stage (IF, ID, EX, MEM, WB) RV32I core with forwarding, stall insertion, branch prediction, misaligned access handling, and debug/commit signal exposure.

The verification environment should automatically load ISA test programs, execute simulations, compare actual architectural state with expected results, and report PASS/FAIL for each test case.

## 1.2. Motivation

The previous labs already worked with RTL models, testbenches, ISA hex files, simulation scripts, and validation outputs, so it is suitable for building an automated verification flow.

Manual verification is not sufficient because RV32I includes arithmetic, logical, load/store, branch, jump, and immediate instructions, and the pipelined design additionally requires hazard and branch behavior verification.

Automated verification improves repeatability and reusability, helps detect regressions after RTL changes, and provides consistent PASS/FAIL reporting across both processor models.

## 1.3. Project Objectives

- Create an automated verification flow for both single-cycle and pipelined RV32I implementations.

- Verify RV32I instruction behavior using directed ISA tests and additional source/ destination operand cases.

Check register write-back, memory read/write behavior, PC updates, branch/jump behavior.

For the pipelined model, verify forwarding, load-use stalls, load-to-branch/jump stalls, branch prediction/flush behavior, and misaligned memory access behavior.

Generate repeatable PASS/FAIL summaries, detailed mismatch logs, waveform outputs, coverage summaries, and performance statistics.

## 1.4. Scope of the Project

- Automated functional verification is the primary scope of this proposal.

The single-cycle core will be used as a simple reference target for ISA correctness.

The pipelined core will be used for advanced tests involving hazards, forwarding, branch prediction, misaligned memory access, and performance counters.

(Optional) Timing closure, FPGA pin assignment validation, physical design, power estimation, and formal verification are outside the initial scope and can be added as future work.

# Block Diagram

Figure 1. Block diagram of the Automated Verification

The proposed automated verification architecture is organized around the Verilog modules and SystemVerilog testbench structure extending with reference checking, coverage collection, and better automated reporting.

## Architecture Description

Makefile

- Automatically generate the ISA file from the ISA assembly file.

- Compile the RTL and testbench files.

- Run the simulation.

Testbench: testbench.sv

- Act as the top-level verification environment.

- Connect the clock/reset generator, test hex file, DUT, tohost poller, and scoreboard.

Clock/Reset Generator: clk_rst_gen

- Generate clk and rst_n signals.

- Provide timing and initialization control for the DUT.

ISA hex file

- Contain the test program instructions.

- Load into the DUT instruction memory, IMEM.

DUT: pipelined.sv

- The hardware design being verified.

- Contain IMEM for instructions and DMEM for data storage.

- Execute the loaded test program.

Tohost Poller

- Monitor dmem_inst.memory[0x800].

- Detects whether the test has passed or failed. If pass, print IPC statistics, display [PASS], and finishes simulation. If fail, call $fatal and report test failure.

Scoreboard

- Monitors DUT signals such as insn_vld, mispred, and ctrl.

- Counts cycles, instructions, control instructions, and mispredictions.

- Calculates performance metrics such as IPC.

Overall Flow

- The Makefile generates the hex file and starts simulation.

- The testbench loads the hex file into the IMEM.

- The DUT executes the program.

- The scoreboard collects performance data.

- The tohost poller determines pass/fail status and ends the simulation.

# Inputs / Outputs

## 3.1. Inputs

| Input Name | Description | Verification Usage |
| --- | --- | --- |
| RISC-V ISA Assembly File (.S/.s) | Assembly test program containing RISC-V instructions to be executed by the CPU. | Provides functional test scenarios to verify instruction execution, control flow, memory access, and pipeline behavior. |
| Makefile | Build an automation script that compiles assembly code, generates HEX files, and launches simulation. | Ensure automated and repeatable verification runs. |
| ISA HEX File (.hex) | Machine-code representation of the assembly program generated by the Makefile. | Loaded into Instruction Memory (IMEM) and used as the instruction stream for the DUT. |
| Clock Signal (clk) | Periodic timing signal generated by clk_rst_gen. | Drives CPU operation and enables cycle-based performance measurement. |
| Reset Signal (rst_n) | Active-low reset signal generated by clk_rst_gen. | Initializes the DUT and verification environment to a known state before testing. |
| DUT (Single-Cycle/ Pipelined RISC-V CPU) | CPU implementation is under verification. | Primary verification targets whose functional correctness and performance are evaluated. |
| Instruction Memory (IMEM) Contents | Program instructions are loaded from the HEX file. | Supplies instructions for execution and enables instruction-related verification. |

## 3.2. Output

| Output Name | Description | Suggested Format |
| --- | --- | --- |
| Pass Indicator (pass_i) | Signal indicating successful completion of the test program. | Boolean signal / Console message ([PASS]) |
| Fail Indicator (fail_i) | Signal indicating test failure detected through the tohost mechanism. | Boolean signal / Fatal error message ([FAIL]) |
| Simulation Log | Execution log generated during simulation. | .log or console output |
| Instruction Count (num_insn) | Total number of valid instructions executed by the DUT. | Integer |
| Cycle Count (num_cycle) | Total clock cycles are consumed during program execution. | Integer |
| Control Instruction Count (num_ctrl) | Number of control-flow instructions executed (branch, jump, etc.). | Integer |
| Branch Misprediction Count (num_mispred) | Number of branch prediction failures detected during execution. | Integer |
| Instructions Per Cycle (IPC) | Performance metric calculated as executed instructions divided by cycle count. | Floating-point value |
| Performance Statistics Report | Summary of execution metrics including cycles, instructions, IPC, and mispredictions. | Console report / Text file |
| Verification Status | Overall test results generated by the scoreboard. | PASS / FAIL |
| Error Report | Information about detected failures, mismatches, or abnormal termination conditions. | Console output / Log file |
| Simulation Termination Status | Final simulation outcome generated by $finish or $fatal. | Exit code and console message |

# Functional Description

| Module | Main Function | Input | Output | Notes |
| --- | --- | --- | --- | --- |
| Test Program | Creates test cases for each instruction, initializes registers/memory, runs instructions, and determines expected results. | Instruction list, operand values, initial state. | Instruction sequence, expected result, PASS/FAIL status. | Include Test ID, seed if random, and scenario description. |
| Testbench | Simulation environment surrounding the DUT; generates clock/reset, loads SRAM, and connects driver/monitor/checker. | Test program, memory image, configuration. | Simulation results, log, waveform. | Include timeout to detect a hanging DUT. |
| Driver | Applies stimulus to the DUT or loads SRAM before simulation starts. | Instruction image, data image, reset/start sequence. | DUT input signals, memory initialization. | Follow the DUT timing/protocol requirements. |
| DUT | The design under test, such as an SC or 5-stage processor. | clk, reset, instruction, memory read data. | PC, register write-back, memory write, status/done. | Do not modify the DUT inside the verification environment. |
| Monitor | Observes DUT behavior without changing DUT operations. | DUT signals, register write-back, memory bus, PC. | Observed transaction, actual result, execution trace. | Log the cycle number and related instruction information. |
| Reference Model | Generates golden results according to the correct instructions of semantics. | Instruction, operands, initial register/memory state. | Expected register/memory/PC/status. | Can be implemented in Python, C, SystemVerilog functions, or an expected table. |
| Scoreboard / Checker | Compare expected and actual results to determine PASS/FAIL. | Observed transaction, golden result, Test ID. | PASS/FAIL, mismatch report, error message. | Log expected vs. actual values for debugging. |
| Coverage Collector | Tracks tested instructions, operands, source/destination combinations, and corner cases. | Test event, instruction type, and operand information. | Coverage percentage, missing bins. | Helps identify missing cases in the test suite. |
| Report Generator | Summarizes simulation results into a readable report. | PASS/FAIL, log, coverage, waveform path. | Summary report, detailed failure report. | Include timestamp, simulator configuration, and regression summary. |

# Verification Strategy

- 5.1. Baseline ISA Regression

Run the existing ISA validation tests already generated and included in the folder as the first regression baseline for further comparison.

Make PASS/FAIL checking automatic and consistent for both DUTs.

Use the existing expected terminal result as smoke-test evidence, then extend the scoreboard for finer-grained architectural checking.

## 5.2. Directed Instruction Tests

Create directed tests for arithmetic, logical, load/store, branch, jump, and upper-immediate instructions.

For each instruction, include zero, positive, negative, maximum/minimum, boundary, and random-like operand values.

Check source/destination register combinations such as rs1 = rd, rs2 = rd, rs1 = rs2, rd = x0, and source = x0 where applicable.

## 5.3. Pipeline-Specific Tests

Verify data forwarding paths using dependent instruction sequences.

Verify load-use hazards and automatic stall insertion.

Verify load-to-branch/jump hazards and required multi-cycle stalls.

Verify branch prediction, BTB behavior, misprediction flushes, and PC recovery.

Collect pipeline performance statistics such as IPC, cycle count, instruction count, branch misprediction rate, stall count, and flush count.

## 5.4. Regression Automation

Use Make-based simulation commands from the previous lab project “Makefile” as the base automation flow.

Add a regression script that runs all selected tests for both DUTs, captures logs, detects PASS/FAIL strings, and stores results in a summary file.

Fail the regression if any test mismatches, times out, or does not produce the expected completion signature.

# PASS/FAIL Criteria

## 6.1. PASS Criteria

The simulation is complete before timeout.

The DUT reaches the expected end-of-test condition and prints or reports TEST PASSED for baseline tests.

Register values, memory values, PC updates, branch/jump results; outputs should match the expected results.

For the pipelined core, hazard handling, forwarding, stalls, flushes, and misaligned access behavior match the expected behavior.

Coverage targets for instruction groups and important source/destination combinations are met.

## 6.2. FAIL Criteria

Any mismatch between expected and actual register, memory, PC, branch behavior.

Unexpected timeout, invalid instruction behavior, incorrect reset behavior, or missing completion signature.

Unknown X/Z values on critical observable signals during active checking.

Pipeline-specific mismatches such as missing forwarding, incorrect stall duration, wrong branch flush, or incorrect misaligned memory result.

# Deliverables

Automated verification on Linux-based environment following the proposal document and references.

Verification plan and test matrix for RV32I instruction groups, operand combinations, and pipeline hazards.

Makefile and tcl scripts for running selected tests on both DUTs.

Enhanced scoreboard/checker logic for architectural-state comparison.

Log and waveform collection flow for debugging failures.

Final report with PASS/FAIL summary, coverage evidence, and selected performance statistics.

# Implementation Plan / Timeline

| Phase | Main Tasks | Expected Output |
| --- | --- | --- |
| Phase 1: Research Topic | Review single_cycle and pipeline_model structure, RTL modules, testbench files, and simulation commands. | Linux-based Verilog automated verification requirements. |
| Phase 2: Verification Matrix | Map RV32I instruction groups, memory operations, I/O checks, and pipeline hazards into a test matrix. | Verification plan and coverage target using SystemVerilog. |
| Phase 3: Baseline Regression | Run generated ISA and demo tests, capture PASS/FAIL status, and define smoke-test criteria. | Baseline regression report. |
| Phase 4: Scoreboard Extension | Add expected-vs-actual checking for PC, register, memory, and selected pipeline debug events. | Automated checker and detailed mismatch logs. |
| Phase 5: Directed and Corner Tests | Add source/destination operand cases, memory boundary cases, branch/jump cases, and pipeline hazard sequences. | Expanded test suite. |
| Phase 6: Coverage and Reporting | Collect instruction, operand, memory, branch, and hazard coverage. Generate final summaries. | Coverage report and final proposal evidence. |
| Phase 7: Final Specification, Demo and Presentation | Prepare demo logs, waveform samples, and final presentation material. | Final project package. |

# Conclusion

- The project provides a strong foundation for automated verification because it includes two RV32I processor architectures, existing validation tests, testbench infrastructure, and simulation automation using SystemVerilog. The proposed project extends this foundation into a structured verification environment that checks instruction correctness, memory I/O behavior, source/destination corner cases, pipeline hazards, and regression quality. The final result will be a repeatable verification flow with clear PASS/FAIL reporting, detailed debug evidence, coverage summaries, and performance statistics.

# References

Andrew Waterman, Yunsup Lee, David A. Patterson, and Krste Asanović, (May 13, 2011). “The RISC-V Instruction Set Manual”. EECS Department, University of California, Berkeley.

Chris Spear, (June 5, 2008). “SystemVerilog for Verification: A Guide to Learning the Testbench Language Features”. Springer.

David A. Patterson, John L. Hennessy, (December 4, 2020). “Computer Organization and Design: The Hardware/Software Interface”. Morgan Kaufmann.

Luciano Lavagno, Grant E. Martin, Igor L. Markov, and Louis K. Scheffer, (2016). “Electronic Design Automation for IC System Design, Verification, and Testing”. CRC Press.