Automated Verification — Team 4

Members: Tran Minh Tuan and Nguyen Minh Duc

Objective

- Create an automated verification flow for both single-cycle and pipelined RV32I implementations.

- Verify RV32I instruction behavior using directed ISA tests and additional source/ destination operand cases.

- Check register write-back, memory read/write behavior, PC updates, branch/jump behavior. For the pipelined model, verify forwarding, load-use stalls, load-to-branch/jump stalls, branch prediction/flush behavior, and misaligned memory access behavior.

- Generate repeatable PASS/FAIL summaries, detailed mismatch logs, waveform outputs, coverage summaries, and performance statistics.

Diagram

Automated Verification Functional Diagram

List of Tasks

Hardware

The project verifies two RV32I processor implementations: a single-cycle RISC-V core and a 5-stage pipelined RISC-V core. The single-cycle processor uses a Harvard architecture with separate instruction and data memories and executes one instruction per cycle, while the pipelined processor uses 5 stages with forwarding, stall handling, branch prediction, flush logic, and debug/commit signal visibility.

Software

The software consists of compact RISC-V assembly test programs that are compiled into HEX files and loaded into instruction memory for simulation. The test program is designed to exercise every implemented RV32I instruction, including arithmetic, logical, load/store, branch, jump, and immediate instructions, while also testing many source and destination register combinations. The program reports the final PASS or FAIL signature through the course tohost mechanism.

Verification Plan

The verification flow uses an automated SystemVerilog testbench, Makefile-based simulation, scoreboard/checker logic, and a tohost poller. The same test program is run on both the single-cycle and pipelined DUTs, then the actual register values, memory values, PC updates, branch behavior, and pipeline-specific events are compared against expected results. The plan also measures both the source-program length and the number of executed instructions to keep the test short while maintaining strong fault-detection capability.

Pipeline-Specific Verification

For the pipelined implementation, the verification focuses on cases that are not visible in a simple single-cycle design, such as data forwarding, load-use stalls, load-to-branch or load-to-jump stalls, branch prediction, misprediction flushes, PC recovery, and misaligned memory access behavior.

Report Out

The final result is a repeatable automated regression flow that runs the test program on both implementations and reports PASS when the expected tohost signature is reached, or FAIL when a mismatch, timeout, missing completion signature, or injected RTL bug is detected. The report includes simulation logs, executed instruction count, cycle count, IPC, control-instruction count, branch-misprediction count, mismatch messages, waveform evidence, and overall PASS/FAIL status.

Fault-Detection Result

The verification program is intended to detect intentional one-line RTL modifications inserted by the TA. If the injected bug changes instruction behavior, register write-back, memory access, PC update, branch handling, stall/flush behavior, or the final program signature, the automated checker and tohost mechanism should report FAIL, proving that the test program has meaningful fault-detection strength despite being compact.