## 1. Expected-versus-actual mismatch reporting

[FAIL][ADDI][TEST_ID=0042]
Cycle       : 37
PC          : 0x00000140
Instruction : 0x00510293
Expected    : x5 = 0x0000000A
Actual      : x5 = 0x00000009
Source      : x2 = 0x00000005
Immediate   : 0x00000005

## 2. Operand, register, and immediate corner cases

x0 used as a source and destination

rd == rs1, rd == rs2, rs1 == rs2, and rd == rs1 == rs2

0x00000000, 0xFFFFFFFF, 0x80000000, and 0x7FFFFFFF

Alternating patterns 0xAAAAAAAA and 0x55555555

Carry, borrow, overflow-boundary, and signed-versus-unsigned comparisons

Immediate values 0, +1, -1, +2047, and -2048

Shift amounts 0, 1, 30, and 31

## 3. Branch, jump, and flush correctness

Separate grouped conditional-branch testing into BEQ, BNE, BLT, BGE, BLTU, and BGEU. For each branch, test taken/not-taken outcomes, forward/backward targets, equal operands, signed boundaries, dependencies, and back-to-back control-flow instructions.

Use poison instructions on the wrong path. If a poison instruction changes a register or memory location, the checker reports a flush failure.

Example

beq  x1, x1, target
addi x20, x0, 1       # Must be flushed
sw   x20, 0(x21)      # Must also be flushed
target:

The testbench should check

Expected x20 = 0x00000000

Actual x20 = 0x00000000

Wrong-path instruction retired = NO

## 4. Load-use and memory corner cases

Load-to-ALU, load-to-branch, load-to-JALR, load-to-store-data, and load-to-store-address

LB versus LBU and LH versus LHU sign/zero extension

SB, SH, and SW byte-enable correctness

Store operations must not modify neighboring bytes

Back-to-back memory operations and load-after-store behavior

Lowest/highest legal addresses and defined alignment behavior

# Recommended Feature Additions

## Priority 1 - Architectural reference model

Track the expected PC, 32 registers, memory state, next PC, and write-back result. Compare DUT and model at retirement. This scales better than a separate checker for every instruction.

[RETIRE] cycle=38 pc=0x00000120 instr=0x005101B3

rd=x3 expected=0x0000000A actual=0x0000000A PASS

## Priority 2 - Functional coverage

Measure opcode, register, immediate, branch outcome, access width, forwarding path, stall reason, flush event, and dependency distance. Add meaningful cross coverage.

================ Coverage Summary ================

Instruction coverage : 100.00%

Branch outcome coverage : 100.00%

Register usage coverage : 93.75%

Hazard coverage : 91.67%

Operand corner coverage : 87.50%

Overall functional coverage: 94.58%

==================================================

## Priority 3 - Constrained-random programs

Generate legal RV32I programs with bounded memory and control-flow targets. Save the random seed and make every failure reproducible.

TEST_NAME = random_rv32i_004

SEED = 18472691

RESULT = PASS

## Priority 4 - Assertions

Check x0 immutability, no retirement after flush, no writes from invalid stages, correct PC holds during stalls, aligned retirement PCs, legal byte enables, and one retirement count per instruction.

## Priority 5 - Regression infrastructure

Run all tests automatically by configuration, return a nonzero exit code for any failure, and generate one consolidated summary.

Example command interface:

The regression report should include:

============ Regression Summary ============

Configuration : RV32I_5_STAGE

Simulator : Xcelium

Total tests : 74

Passed : 74

Failed : 0

Timed out : 0

Assertion failures : 0

Functional coverage : 94.58%

Total cycles : 18240

Result : PASS

============================================

## Priority 6 - Timeout/deadlock detection

Stop tests that fail to reach tohost within a configurable cycle budget and print the last PC and pipeline state.

[TIMEOUT] Test did not write to TOHOST within 10000 cycles

Last retired PC : 0x00000218

Last fetched PC : 0x00000224

Pipeline status : IF=valid ID=valid EX=stalled MEM=valid WB=invalid

## Priority 7 - Failure artifacts

Save logs, failing seeds, coverage, and waveforms. A useful optimization is to rerun only failed tests with waveform dumping enabled.

logs/addi.log

logs/hazard_load_use.log

waves/hazard_load_use_fail.fsdb

reports/regression_summary.txt

reports/coverage.html

seeds/failing_seeds.txt

# Recommended Reporting Formats

## 1. Regression summary

============ Regression Summary ============
Configuration       : RV32I_5_STAGE
Simulator           : Xcelium
Total tests         : 74
Passed              : 74
Failed              : 0
Timed out           : 0
Assertion failures  : 0
Functional coverage : 94.58%
Total cycles        : 18240
Result              : PASS
============================================

## 2. Coverage summary

================ Coverage Summary ================
Instruction coverage       : 100.00%
Branch outcome coverage    : 100.00%
Register usage coverage    : 93.75%
Hazard coverage            : 91.67%
Operand corner coverage    : 87.50%
Overall functional coverage: 94.58%
==================================================

## 3. Expanded performance report

Total cycles                = 260
Retired instructions        = 230
IPC                         = 0.8846
CPI                         = 1.1304
Conditional branches        = 30
JAL / JALR                  = 7 / 5
Mispredictions              = 12
Load-use stall cycles       = 6
Other stall cycles          = 8
Flush events                = 12
Flushed instructions        = 18

# Verify Both DUT Configurations

Both single-cycle and five-stage pipelined DUT should be supported, the final evidence should contain separate, clearly labeled runs.

| Evidence item | Single-cycle DUT | Five-stage pipeline DUT |
| --- | --- | --- |
| Configuration label | Required | Required |
| ISA and corner tests | Required | Required |
| Architectural scoreboard | Required | Required |
| Forwarding checks | Not applicable | Required |
| Load-use stalls | Not applicable | Required |
| Flush/recovery checks | Control-flow result only | Required |
| Performance report | Cycles, instructions, IPC/CPI | Plus stalls, flushes, forwarding, redirects |

[CONFIG] DUT_TYPE=SINGLE_CYCLE
Result : PASS
IPC    : 1.00

[CONFIG] DUT_TYPE=PIPELINE_5_STAGE
Result       : PASS
IPC          : 0.88
Stall cycles : 14
Flush cycles : 16

# Final Requirement Matrix

| Topic 7 requirement | Current evidence | Assessment |
| --- | --- | --- |
| Automated execution | Yes | Meets |
| Automated PASS/FAIL | Yes | Meets |
| Directed ISA tests | Yes | Meets |
| Pipeline hazard tests | Yes | Mostly meets |
| Operand/register corner cases | Not clearly shown | Needs evidence |
| Forwarding checks | Indicated | Needs finer breakdown |
| Load-use stall checks | Not explicit | Needs improvement |
| Branch/jump behavior | Yes | Mostly meets |
| Flush behavior | Not explicitly proven | Needs improvement |
| Expected-versus-actual logs | Not shown | Missing from evidence |
| Repeatable regression | Likely | Needs evidence |
| Waveform generation | Not shown | Needs evidence |
| Coverage summary | Not shown | Missing from evidence |
| Performance statistics | Yes; consistent | Meets |
| Single-cycle and pipeline runs | Claimed; not separately labeled | Needs evidence |